# Comprehensive Biosensor Catalog

NeuroPlay Platform Integration Guide

December 2025 | Prepared for AuraSense Tech Corporation

# Executive Summary

This catalog provides an exhaustive inventory of commercially available biosensors across all human physiological modalities relevant to the NeuroPlay real-time neurological assessment platform. Each device is evaluated for API availability, data specifications, integration pathways, and clinical validation status.

# 1. In-Ear Neurophysiology (EEG/EMG/EOG/EDA)

Revolutionary form factor placing clinical-grade neurophysiological sensors in truly wireless earbuds. The ear canal provides excellent signal quality due to proximity to temporal cortex and rich vascularization.

## NextSense (Tone by NextSense)

Mountain View, CA | nextsense.io | tonebynextsense.com

| Parameter | Specification |
| --- | --- |
| Signals | 6-channel in-ear EEG, EMG (jaw clench), EOG (eye movement), motion sensors |
| Technology | Clinical-grade EEG in truly wireless earbuds; originated at Google X |
| Validation | 86.4% seizure detection accuracy vs clinical gold standard (1,255 hours, 20 epilepsy patients); Bioelectron Med 2024 |
| Features | Adaptive audio responds to brain states (sleep/focus/relax); auto-pause on sleep detection |
| Launch | Consumer brand 'Tone' debuted CES 2025; Q4 2025 availability |
| Funding | $16M Series A (Nov 2025); $24.34M total raised |
| API Status | Not yet public; research/pharma partnerships in development for 2027 |

## IDUN Guardian

Glattpark, Switzerland | iduntechnologies.com

| Parameter | Specification |
| --- | --- |
| Signals | In-ear EEG, EMG, EOG from single channel (polysomnography-equivalent) |
| Technology | Patented DRYODE dry electrode technology (no gels/caps); ADI analog front-end |
| Features | Frequency band classifiers, jaw clench detection, eye movement detection, skin contact impedance, pacemaker detection |
| Connectivity | Bluetooth LE |
| API Status | Cloud-based neuro-intelligence platform API available |
| Founded | 2017 |

# 2. Electrodermal Activity (EDA/GSR)

Electrodermal activity measures skin conductance changes driven by sympathetic nervous system activation. Key for stress detection, emotional arousal, and cognitive load assessment.

## Empatica EmbracePlus (successor to E4)

Boston, MA | empatica.com

| Parameter | Specification |
| --- | --- |
| Signals | EDA (4 Hz), PPG heart rate, skin temperature, 3-axis accelerometer |
| Form Factor | Wrist-worn clinical-grade research wearable |
| Validation | Cited in 45,700+ peer-reviewed publications; 84% stress detection accuracy |
| API Status | Research API available; Bluetooth streaming; cloud platform |
| Note | E4 retired; EmbracePlus is current research platform |

## Shimmer GSR3+

Dublin, Ireland | shimmersensing.com

| Parameter | Specification |
| --- | --- |
| Signals | GSR/EDA, PPG, accelerometer; finger electrode placement (gold standard) |
| Range | Higher µS range than wrist devices (3-5x Empatica amplitude) |
| Placement | Fingers (highest signal), foot, wrist options |
| API Status | Full SDK; Bluetooth streaming; widely used in research |

# 3. Continuous Glucose Monitoring (CGM)

CGM provides real-time glucose trends for metabolic state assessment. Glucose variability correlates with cognitive performance, energy levels, and stress response.

## Dexcom G7/G6

San Diego, CA | dexcom.com | developer.dexcom.com

| Parameter | Specification |
| --- | --- |
| Signals | Real-time glucose readings, trends, alerts (5-minute intervals) |
| Sensor Life | Up to 15 days (G7); ~73.9% last full duration |
| API Status | FDA-cleared Partner Web APIs (OAuth 2.0); invite-only developer program |
| Integration | Garmin smartwatches, Epic EHR, insulin pumps |

## Abbott FreeStyle Libre 3

Abbott Park, IL | diabetescare.abbott

| Parameter | Specification |
| --- | --- |
| Signals | Continuous glucose, trends; smallest CGM sensor available |
| Sensor Life | 14 days |
| API Status | LibreView cloud API via aggregators (Thryve, Terra, Validic); unofficial APIs available |
| Integration | Insulin pumps (Tandem, Omnipod, mylife Loop), NovoPen smart pens |

# 4. Muscle Oxygen (NIRS SmO2)

Near-infrared spectroscopy measures muscle oxygen saturation (SmO2) for real-time assessment of metabolic state, fatigue, and recovery. Critical for exercise physiology and performance optimization.

## Moxy Monitor

Fortiori Design LLC, Hutchinson, MN | moxymonitor.com

| Parameter | Specification |
| --- | --- |
| Signals | SmO2 (0-100% absolute), tHb (total hemoglobin) |
| Technology | CW-NIRS with adipose tissue layer correction (Monte Carlo model) |
| Sample Rate | 0.5, 1, or 2 Hz output (samples at 1000+ Hz internally) |
| Validation | 24% of 191 sports science studies; CV 10% reliability |
| Users | Olympic/World Champion athletes (Kristian Blummenfelt) |
| API Status | ANT+ streaming; Bluetooth; Garmin/Wahoo integration |

## Artinis PortaMon / Train.Red

Artinis Medical Systems, Netherlands | artinis.com | train.red

| Parameter | Specification |
| --- | --- |
| Signals | O2Hb, HHb, tHb, TSI% (tissue saturation index) |
| Validation | 40% of 191 sports science studies; most-cited NIRS device |
| Sample Rate | Train.Red: 10-25 Hz (100 Hz planned); PortaMon: 100 Hz |
| API Status | Bluetooth wireless; OxyMon software; research SDK |

# 5. Consumer EEG Headsets

Consumer-grade EEG devices provide accessible brain activity monitoring. While lower channel count than research systems, validated for specific applications including BCI, meditation, and drowsiness detection.

## Emotiv EPOC-X / EPOC Flex

San Francisco, CA | emotiv.com

| Parameter | Specification |
| --- | --- |
| Channels | 14 channels (EPOC-X); 32 channels (EPOC Flex) |
| Sample Rate | 128 Hz or 256 Hz |
| Electrodes | Saline-wet sponge contacts (gold electrodes) |
| Research Use | 67.69% of 916 consumer EEG studies; most widely used |
| API Status | EmotivPRO API ($89/month subscription); BrainFlow compatible |

## InteraXon Muse S / Muse 2

Toronto, Canada | choosemuse.com

| Parameter | Specification |
| --- | --- |
| Channels | 4 EEG channels (AF7, AF8, TP9, TP10) + reference (FPz) |
| Electrodes | Silver ink (forehead) + conductive silicone-rubber (behind ear) |
| Form Factor | Muse S: Soft fabric headband (sleep-friendly) |
| API Status | BrainFlow compatible; third-party SDKs; meditation app ecosystem |

## OpenBCI Cyton/Ganglion

Brooklyn, NY | openbci.com

| Parameter | Specification |
| --- | --- |
| Channels | Cyton: 8 channels (daisy-chain to 16); Ganglion: 4 channels |
| Sample Rate | 125 Hz (Ganglion) to 250 Hz (Cyton) |
| Flexibility | Fully customizable electrode placement; EEG, EMG, ECG capable |
| API Status | Open-source; OpenBCI GUI; BrainFlow native support |
| Price Range | $1,000-$2,000 complete system |

# 6. Respiration Monitoring

Respiratory rate and patterns provide insight into autonomic state, stress, cognitive load, and physical exertion. Key parameter for comprehensive physiological assessment.

## Hexoskin Smart Shirts

Carré Technologies, Montreal, Canada | hexoskin.com

| Parameter | Specification |
| --- | --- |
| Signals | ECG, breathing rate (RPM), minute ventilation (L/min), chest/abdominal RIP sensors |
| Technology | Respiratory inductance plethysmography (RIP) textile sensors |
| Validation | 275+ scientific publications; strong correlation with reference standards |
| API Status | Connected Health Platform; research API available |

## Zephyr BioHarness 3 (Medtronic)

Medtronic/Zephyr Technology | zephyranywhere.com

| Parameter | Specification |
| --- | --- |
| Signals | ECG, HR, HRV, respiration rate, skin temperature, posture, activity (3-axis accelerometer) |
| Form Factor | Chest strap with detachable module; compression shirts available |
| Applications | Military, first responder, occupational health, sports science |
| Price | ~£798 (module only) |
| API Status | Bluetooth BTLE; SDK available; ProRAE Guardian integration |

# 7. Cuffless Blood Pressure

Continuous, non-invasive blood pressure monitoring is a breakthrough technology. The first FDA-cleared OTC cuffless BP monitor was approved in July 2025.

## Aktiia Hilo Band (G0 System)

Neuchâtel, Switzerland | aktiia.com | FDA 510(k) cleared July 2025

| Parameter | Specification |
| --- | --- |
| Technology | PPG optical sensors; pulse wave analysis; AI algorithms |
| Accuracy | ±5 mmHg vs invasive arterial line (exceeds FDA thresholds) |
| Calibration | Integrated oscillometric cuff (monthly calibration); CALFREE technology in development |
| Regulatory | FIRST FDA-cleared OTC cuffless BP monitor (July 2025) |
| Users | 130,000+ globally |
| US Launch | 2026; ~$199 expected |
| Funding | $42M Series B (May 2025); $100M+ total raised |

# 8. Core Body Temperature

Continuous core body temperature monitoring enables thermoregulation optimization, heat training, and circadian rhythm assessment. Critical for athletic performance and occupational safety.

## greenTEG CORE Sensor

greenTEG AG, Zurich, Switzerland | corebodytemp.com | greenteg.com

| Parameter | Specification |
| --- | --- |
| Technology | Heat flux measurement; AI-powered calera® algorithm |
| Validation | 10+ clinical studies; validated against ingestible pills |
| Form Factor | Waterproof sensor; chest strap or HR monitor attachment |
| Integration | Garmin, COROS, Suunto, Wahoo, Hammerhead compatibility |
| Battery | 15 days (with HR monitor pairing for best accuracy) |
| OEM | calera® technology available for white-label integration |

# 9. Research-Grade Acquisition Systems

Gold-standard laboratory systems for multi-channel physiological data acquisition. Used in 99% of top universities worldwide.

## BIOPAC MP200/MP160

BIOPAC Systems, Goleta, CA | biopac.com

| Parameter | Specification |
| --- | --- |
| Channels | 16 channels (expandable); differing sample rates per channel |
| Sample Rate | Up to 100 kHz/channel (1,600 kHz aggregate for MP200) |
| Signals | ECG, HRV, EEG, EMG, EGG, EOG, EDA, respiration, blood pressure, and 40+ research fields |
| Citations | 54,700+ peer-reviewed publications; 99% of top universities |
| Software | AcqKnowledge with automated analysis routines |
| API Status | Network Data Transfer (NDT); BHAPI hardware API; ACKAPI software API |

# 10. NeuroPlay Integration Architecture

Recommended tiered approach for biosensor integration into the NeuroPlay platform, prioritizing real-time streaming capability and clinical validation.

## Recommended Tier Structure

| Tier | Devices | Integration Pathway |
| --- | --- | --- |
| Core | Joy-Con, DualSense, etee VR | Native WebHID/Bluetooth; existing SDK |
| Tier 1 | Polar H10 (ECG/HRV) | Bluetooth LE; Polar SDK; real-time streaming |
| Tier 2 | EmotiBit, BITalino (EDA/PPG) | WiFi/Bluetooth; open-source SDK; real-time |
| Tier 3 | uMyo (EMG) | Bluetooth LE; real-time muscle activity |
| Tier 4 | Muse, OpenBCI (EEG) | BrainFlow unified SDK; real-time |
| Tier 5 | NextSense, IDUN (in-ear) | API pending (2027); partnership required |
| Tier 6 | Dexcom G7 (CGM) | Partner API (invite-only); 5-min polling |
| Tier 7 | Moxy, CORE (SmO2/Temp) | ANT+/Bluetooth; real-time |
| Passive | WHOOP, Oura | Daily recovery context; cloud API |

## Unifying SDK: BrainFlow

BrainFlow provides a unified C/C++/Python/Rust API for neurophysiological data acquisition across OpenBCI, Muse, NeuroSky, BrainBit, Emotiv, and other EEG/EMG devices. Recommended as the primary integration layer for consumer neurophysiology sensors.

## API Aggregation Platforms

- Thryve: 500+ health sources including Dexcom, FreeStyle Libre; OAuth-based; GDPR/HIPAA/ISO 27001

- Terra API: Wearable aggregation; FreeStyle Libre, Garmin, Oura, WHOOP; webhook delivery

- Validic: Enterprise health data platform; Abbott integration

# Document Information

Prepared by: Claude (Anthropic) for AuraSense Tech Corporation

Date: December 16, 2025

Version: 1.0

Classification: Internal Use - NeuroPlay Development

Note: API availability, pricing, and specifications subject to change. Verify current status with manufacturers before integration.