# Delta-Predictive Biosensing

Neuromorphic Simulation Framework & Public Dataset Catalog

Hand-Contact Physiological Measurement System

# Executive Summary

This document provides a comprehensive framework for validating Delta-Predictive Biosensing (DPB) using neuromorphic computing and spiking neural networks. The framework inverts the traditional measurement paradigm: instead of capturing raw signals and extracting features, DPB measures deviations from expected population-level templates, enabling rapid, sparse measurements suitable for non-compliant populations.

Key Innovation: Humans are remarkably similar. By leveraging strong priors based on population physiology, measurements converge in 2-5 seconds instead of 30-60 seconds, with 100x reduction in power consumption and robust tolerance to motion artifacts.

# Section 1: Public Biosignal Dataset Catalog

The following datasets are publicly available for validating the DPB framework. Each dataset includes ground truth physiological measurements that can be used to generate synthetic sparse event streams and validate reconstruction accuracy.

## 1.1 ECG & PPG Datasets

| Dataset | Signals | Subjects/Records | Access |
| --- | --- | --- | --- |
| PTB-XL | 12-lead ECG | 21,799 ECGs, 18,869 patients | physionet.org/content/ptb-xl |
| MIT-BIH Arrhythmia | 2-lead ECG | 48 records, 30 min each | physionet.org/content/mitdb |
| BUT PPG | PPG, ECG, ACC | 3,888 10s recordings | physionet.org/content/butppg |
| Pulse Transit Time PPG | Multi-site PPG, ECG, IMU | 22 subjects, 66 recordings | physionet.org/content/pulse-transit-time-ppg |
| Wrist PPG During Exercise | PPG, ACC, Gyro, ECG ref | Walk/run/bike sessions | physionet.org/content/wrist |
| ScientISST MOVE | ECG, EMG, EDA, PPG, Temp, ACC | Daily activities, naturalistic | physionet.org/content/scientisst-move |
| BIDMC PPG & Respiration | PPG, Respiration | 53 ICU records | physionet.org/content/bidmc |
| ECG-PPG Simulator | Synthetic ECG/PPG | Configurable arrhythmias | physionet.org/content/ecg-ppg-simulator-arrhythmia |

## 1.2 EDA/GSR (Electrodermal Activity) Datasets

| Dataset | Signals | Subjects/Records | Access |
| --- | --- | --- | --- |
| WESAD | EDA, BVP, ECG, EMG, Resp, Temp, ACC | 15 subjects, stress/affect states | archive.ics.uci.edu/dataset/465 |
| CASE (Continuous Affect) | EDA, ECG, continuous annotations | Emotion recognition | Available via request |
| CLAS | EDA, ECG, PPG | Cognitive load assessment | GitHub: cognitive-load-assessment |
| VerBIO | EDA, ECG, PPG | Stress detection | Available via request |
| PhyAAt (Auditory Attention) | EEG, GSR, PPG | 25 subjects, 128 Hz | GitHub: PhyAAt |
| SWELL | EDA, HR, facial, body | 25 subjects, work stress | cs.ru.nl/~skoldijk/SWELL-KW |
| Empatica E4 Stress Dataset | EDA, PPG (E4 wristband) | 29 subjects | Mendeley Data |

## 1.3 EMG (Electromyography) Datasets

| Dataset | Details | Subjects/Gestures | Access |
| --- | --- | --- | --- |
| NinaPro DB1 | 10 electrodes, 100 Hz | 27 subjects, 52 gestures | ninapro.hevs.ch |
| NinaPro DB2 | 12 electrodes (Delsys Trigno), 2 kHz | 40 subjects, 52 gestures | ninapro.hevs.ch |
| NinaPro DB5 | 16 electrodes, 200 Hz | 10 subjects, 52 gestures | ninapro.hevs.ch |
| putEMG | 24 electrodes, 5.12 kHz | 44 subjects, 8 gestures, 20 reps | biolab.put.poznan.pl/putemg-dataset |
| Multi-day EMG | Forearm + wrist, multiple days | 43 subjects, Days 1, 8, 29 | nature.com/articles/s41597-022-01836-y |
| BioPatRec | 8 electrodes | Multiple DBs available | github.com/biopatrec |
| SEEDS | HD-EMG, data glove | 25 subjects, 13 movements | Available via request |

## 1.4 Tremor & Parkinson's Disease Datasets

| Dataset | Modality | Subjects | Access |
| --- | --- | --- | --- |
| mPower | ACC, gyro, voice, tapping | 9,520 subjects (1,087 PD) | synapse.org/mPower |
| UCI Parkinson's Spiral | Graphics tablet: x,y,z,pressure,time | 62 PD + 15 healthy | archive.ics.uci.edu/dataset/395 |
| Improved Spiral Test | Static/Dynamic spiral, stability | 25 PD + 15 healthy | archive.ics.uci.edu/dataset/358 |
| Levodopa Response Study | Wrist ACC (GeneActiv), 50 Hz | 31 PD, 4-day protocol | nature.com/articles/s41597-021-00830-0 |
| PADS (Smartwatch) | ACC + gyro (2 smartwatches) | PD + other disorders + HC | nature.com/articles/s41531-024-00826-6 |
| REMAP | Wrist ACC, skeleton pose video | PD on/off meds + controls | nature.com/articles/s41597-023-02663-5 |
| UCI Parkinson's Voice | Voice biomarkers (dysphonia) | 31 subjects, 195 recordings | archive.ics.uci.edu/dataset/174 |
| UCI Parkinson's Telemonitoring | Voice features, UPDRS scores | 42 subjects, 5,875 recordings | archive.ics.uci.edu/dataset/189 |

## 1.5 Multimodal Physiological Datasets

| Dataset | Signals | Context | Access |
| --- | --- | --- | --- |
| Stress Recognition (Driving) | ECG, EMG, GSR, Respiration | Driving stress | physionet.org/content/drivedb |
| CEBS | 2x ECG, Respiration, SCG | 60 records, 20 volunteers | physionet.org/content/cebsdb |
| Cerebral Vasoregulation (Stroke) | ECG, BP, EMG, ACC, gait pressure | 60 stroke + 60 control | physionet.org/content/cves |
| Non-EEG Neurological Status | ACC, gyro, grip force | Neurological assessment | physionet.org/content/noneeg |
| MIMIC-III Waveform | ECG, ABP, PPG, Resp, SpO2 | ICU patients | physionet.org/content/mimiciii-waveform |
| Hexoskin Research | ECG, Resp rate, minute vent | Activity monitoring | hexoskin.com/pages/data-sharing |

## 1.6 Bioimpedance Datasets

Bioimpedance measurement datasets include impedance cardiography (ICG), electrical impedance tomography (EIT), and body composition analysis (BIA). These provide validation data for impedance-based physiological monitoring through handheld objects.

### Impedance Cardiography (ICG)

| Dataset | Signals | Subjects | Access |
| --- | --- | --- | --- |
| HeartCycle | ICG, ECG, PPG, ECHO, Heart Sounds (synced) | 17 healthy | physionet.org/content/heartcycle |
| CEBS | 2x ECG, Resp, Seismocardiogram | 20 volunteers | physionet.org/content/cebsdb |

### Bioimpedance for Hydration/Dehydration

| Dataset | Signals | Subjects | Access |
| --- | --- | --- | --- |
| Quantitative Dehydration | Bioimpedance, Temp, Salivary, Sweat | Multiple | physionet.org/content/qde |
| Graphene Bioimpedance Tattoos | 4-ch Wrist Bioimpedance, BP, PPG | 7 | physionet.org (graphene-bp) |
| BIDMC PPG + Respiration | PPG, Impedance Resp, Breath annot. | 53 ICU | physionet.org/content/bidmc |

### Electrical Impedance Tomography (EIT)

| Dataset | Description | Access |
| --- | --- | --- |
| UEF 2D EIT (2017) | Circular tank phantom, CC BY 4.0 | fips.fi/open-datasets/eit-datasets |
| KTC2023 (Kuopio Challenge) | Real-world EIT measurements | zenodo.org/records/8252370 |
| EIDORS Simulated | Synthetic thorax/lung EIT data | github.com/shuaikaishi/CDEIT |
| Stroke EIT Dataset | Brain imaging EIT data | GitHub (EIT topics) |

### Body-Coupled Communication (BCC)

| Dataset | Subjects | Signals | Access |
| --- | --- | --- | --- |
| Galvanic-Coupled BCC Signal Loss | 30 | Signal loss + bioimpedance | ScienceDirect (Data in Brief) |

Note: The BIA International Database (whole-body composition with raw R, Xc, phase angle data) is currently in development at University of Lisbon. HeartCycle is particularly valuable for DPB validation as it provides synchronized ICG and ECHO, enabling hemodynamic parameter validation (stroke volume, cardiac output, PEP, LVET).

# Section 2: Framework Architecture

## 2.1 Core Components

The DPB simulation framework consists of five interconnected modules:

- Dataset Loaders: Unified interface for loading PhysioNet, UCI, and custom datasets

- Prior Models: Population-level statistical models for ECG, PPG, EDA, EMG, tremor

- Event Encoders: Level-crossing ADC, template deviation, derivative-based spike generation

- SNN Inference: Leaky integrate-and-fire networks for sparse event decoding

- Evaluation Metrics: Convergence time, accuracy vs. contact duration, power estimation

## 2.2 Software Dependencies

| Package | Version | Purpose |
| --- | --- | --- |
| snntorch | >=0.9.1 | Spiking neural network framework (PyTorch-based) |
| spikingjelly | >=0.0.0.0.14 | Alternative SNN framework with CUDA acceleration |
| neurokit2 | >=0.2.7 | Biosignal processing (ECG, PPG, EDA, EMG) |
| wfdb | >=4.1.2 | PhysioNet WFDB file format support |
| scipy | >=1.11.0 | Signal processing, Bayesian filtering |
| torch | >=2.0.0 | Deep learning backend |
| numpy | >=1.24.0 | Numerical computing |
| pandas | >=2.0.0 | Data manipulation |

## 2.3 Signal-to-Modality Mapping

The following table maps hand-contact object types to measurable physiological signals:

| Object Type | Contact Type | Primary Signals | Recommended Datasets |
| --- | --- | --- | --- |
| Pen/Stylus | Precision grip | Tremor, EDA, fine motor | UCI Spiral, mPower tapping |
| Game Controller | Two-handed grip | ECG, EDA, EMG, HRV | WESAD, NinaPro DB2 |
| Utensil (Spoon) | Precision/power hybrid | Tremor, bradykinesia | Levodopa Response, PADS |
| Toothbrush | Power grip, repetitive | Tremor, motor patterns | mPower, REMAP |
| Remote Control | Lateral pinch, brief | EDA phasic, reaction time | WESAD, Stress Recognition |
| Cup/Mug | Cylindrical grip | Tremor (liquid amplified), temp | UCI Spiral, PADS |
| Steering Wheel | Two-handed sustained | ECG, EDA, drowsiness | Stress Recognition (Driving) |
| Walking Stick | Power grip + gait | Gait, tremor, balance | REMAP, Levodopa Response |

# Section 3: Python Implementation

The following code provides the core simulation framework. Full implementation is provided in the accompanying Python package.

## 3.1 Event Generation (Sparse Encoding)

Level-Crossing ADC Simulation:

class LevelCrossingADC:

def __init__(self, delta_v, v_ref=0):

self.delta_v = delta_v

self.current_level = int(v_ref / delta_v)

def process(self, signal, timestamps):

events = []

for v, t in zip(signal, timestamps):

new_level = int(v / self.delta_v)

if new_level != self.current_level:

polarity = 1 if new_level > self.current_level else -1

events.append({'t': t, 'pol': polarity})

self.current_level = new_level

return events

## 3.2 Prior Model (Population Statistics)

class HeartRatePrior:

"""Population-level HR prior for delta-predictive inference"""

def __init__(self, mean_hr=75, std_hr=12, tau_drift=30):

self.mean_hr = mean_hr

self.std_hr = std_hr

self.tau = tau_drift  # seconds

# State: [log_hr, d_log_hr/dt]

self.state = np.array([np.log(mean_hr), 0])

self.P = np.diag([0.15**2, 0.05**2])

def predict(self, dt):

decay = np.exp(-dt / self.tau)

self.state[0] = decay * self.state[0] + (1-decay) * np.log(self.mean_hr)

return np.exp(self.state[0])

## 3.3 SNN Decoder Architecture

import snntorch as snn

import torch.nn as nn

class DeltaPredictiveSNN(nn.Module):

def __init__(self, n_input=64, n_hidden=256, n_output=8):

super().__init__()

# Input: sparse event stream

self.fc1 = nn.Linear(n_input, n_hidden)

self.lif1 = snn.Leaky(beta=0.9)  # fast time constant

self.fc2 = nn.Linear(n_hidden, n_hidden)

self.lif2 = snn.Leaky(beta=0.95)  # medium time constant

self.fc3 = nn.Linear(n_hidden, n_output)

# Output: [HR, HRV, EDA_tonic, EDA_phasic, tremor_4-6, tremor_6-12, grip, confidence]

# Section 4: Validation Protocol

## 4.1 Experimental Design

For each dataset and signal type, the validation protocol consists of:

- Ground Truth Extraction: Extract reference physiological parameters from full continuous signals using established algorithms (Pan-Tompkins for ECG, cvxEDA for EDA, etc.)

- Event Stream Generation: Convert continuous signals to sparse event streams using level-crossing ADC simulation with varying quantization levels

- Delta-Predictive Inference: Apply Bayesian filtering with population priors to reconstruct physiological parameters from sparse events

- Convergence Analysis: Measure time-to-convergence (uncertainty below clinical threshold) across varying contact durations

- Comparison: Compare DPB performance against traditional fixed-rate sampling with feature extraction

## 4.2 Expected Outcomes by Modality

| Signal | Traditional Time | DPB Time | Clinical Threshold | Dataset |
| --- | --- | --- | --- | --- |
| Heart Rate | 10-30 sec | 2-3 sec | ±5 BPM | PTB-XL |
| HRV (RMSSD) | 5 min | 30-60 sec | ±15 ms | MIT-BIH |
| EDA Tonic | 10-30 sec | 3-5 sec | ±1 µS | WESAD |
| EDA Phasic | Event-dependent | 0.5-2 sec | Detection | WESAD |
| Tremor (PD) | 10-30 sec | 3-5 sec | ±0.5 Hz, classification | mPower |
| Tremor (ET) | 10-30 sec | 2-4 sec | ±1 Hz | UCI Spiral |
| Grip Force | 2-5 sec | 0.5-1 sec | ±10% MVC | NinaPro |
| EMG Gesture | 0.5-2 sec | 0.2-0.5 sec | Classification | NinaPro DB2 |

## 4.3 Power Consumption Estimation

Power advantage comes from three sources:

- ADC Power: Level-crossing ADC generates 10-100x fewer samples than Nyquist-rate ADC

- Processing Power: SNN inference is event-driven; computation only occurs when spikes arrive

- Transmission Power: Sparse event streams require 10-100x less bandwidth to transmit

Estimated power comparison (for continuous heart rate monitoring):

| Component | Traditional | DPB |
| --- | --- | --- |
| ADC (1 channel) | 100-500 µW | 1-10 µW |
| MCU Processing | 1-10 mW | 10-100 µW |
| BLE Transmission | 10-50 mW (burst) | 1-5 mW (sparse) |
| Total (Active) | 10-60 mW | 100 µW - 1 mW |

# Section 5: Neuromorphic Computing Resources

## 5.1 SNN Software Frameworks

| Framework | Backend | Key Features | Best For |
| --- | --- | --- | --- |
| snnTorch | PyTorch | Surrogate gradients, tutorials | Research, prototyping |
| SpikingJelly | PyTorch + CUDA | CuPy backend, 11x speedup | Large-scale training |
| Norse | PyTorch | Functional design, torch.compile | Custom neuron models |
| Lava (Intel) | Custom + Loihi | Hardware deployment | Loihi neuromorphic chip |
| Brian2 | NumPy/C++ | Equations-based, biological | Computational neuroscience |
| BindsNET | PyTorch | STDP learning rules | Unsupervised learning |

## 5.2 Neuromorphic Hardware Targets

| Chip | Manufacturer | Power | Application |
| --- | --- | --- | --- |
| Loihi 2 | Intel | ~1 W | Research, complex inference |
| Akida | BrainChip | 1-30 mW | Edge AI, ultra-low power |
| Xylo | SynSense | <1 mW | Biosignal processing |
| µBrain | IMEC | <100 µW | Wearables, implants |
| Speck | SynSense | 0.7 mW | Event camera processing |

## 5.3 Biosignal Processing Libraries

- NeuroKit2: Comprehensive biosignal processing (ECG, PPG, EDA, EMG, RSP)

- BioSPPy: Signal processing for physiological signals

- HeartPy: Heart rate analysis from PPG and ECG

- cvxEDA: Convex optimization for EDA decomposition

- WFDB: PhysioNet waveform database toolkit

- MNE-Python: EEG/MEG analysis (useful for tremor spectral analysis)

# Section 6: Implementation Roadmap

## 6.1 Phase 1: Simulation Validation (Weeks 1-4)

- Download and preprocess PTB-XL (ECG), WESAD (EDA), mPower (tremor), NinaPro DB2 (EMG)

- Implement level-crossing ADC simulator with configurable quantization

- Build population prior models from training splits

- Train SNN decoders using snnTorch with surrogate gradient descent

- Validate convergence time and accuracy on held-out test sets

## 6.2 Phase 2: Hardware Prototype (Weeks 5-12)

- Design analog front-end with level-crossing comparators (AD8232 + comparator array)

- Integrate BrainChip Akida or SynSense Xylo development kit

- Build instrumented pen/utensil prototype with EDA + IMU sensors

- Port trained SNN models to neuromorphic hardware

- Measure real-world power consumption and latency

## 6.3 Phase 3: Clinical Validation (Weeks 13-24)

- IRB approval for human subjects testing

- Recruit participants including non-compliant populations (PD, pediatric, neurodivergent)

- Compare DPB measurements against gold-standard references (BIOPAC, clinical ECG)

- Document usability and acceptance in target populations

- Prepare regulatory pathway documentation (if applicable)

— End of Document —